# AgriTwin-GH Report

1. Project Overview

AgriTwin-GH is an AI-driven, IoT-enabled digital twin system designed for greenhouse cultivation of tomato and strawberry.
The system integrates real-time sensor data, computer vision, machine learning, and predictive control to:

- Optimize resource utilization (energy, water, fertilizer, CO₂)

- Maintain plant health and yield quality

- Predict and manage diseases, growth stages, and harvest timing

- Operate safely and intuitively for illiterate or semi-literate greenhouse workers

2. Disease Classification & Temporal Disease Progression

2.1 Base Repository Used (CNN)

GitHub repository used for disease classification:

https://github.com/ravikant-diwakar/AgriSens

From this repository, only the disease classification module is used.

- The repository uses CNN (Convolutional Neural Networks) for static image-based disease classification

- The CNN architecture is reused and adapted

2.2 Target Diseases

Strawberry

- Powdery Mildew

- Leaf Scorch

Tomato

- Powdery Mildew

- Leaf Mold

- Spider Mites

2.3 Extension Beyond the Repository (Key Contribution)

The project extends beyond static classification:

- CNN → detects disease from single images

- RNN → analyzes disease growth patterns over time

- Enables:

  - Disease progression tracking

  - Future disease impact prediction

  - Risk-aware greenhouse control decisions

This allows the system to move from detection → prediction → prevention.

3. Greenhouse Factors Considered (Tomato & Strawberry)

3.1 Environmental Factors

- Air temperature

- Relative humidity

- Light intensity & photoperiod

- CO₂ concentration

- Ventilation rate

- Air circulation

- Leaf wetness

3.2 Growth & Crop Factors

- Soil pH

- Electrical Conductivity (EC – nutrient concentration)

- Growth stage (vegetative, flowering, fruiting, harvest)

3.3 Disease & Stress Factors

- High humidity duration

- Nutrient imbalance (e.g., calcium deficiency → blossom end rot)

- Sensor drift or abnormal readings

4. Sensors Used and Factor Mapping

4.1 Climate Sensors

- Air temperature → DS18B20, DHT22

- Relative humidity → DHT22, SHT31

- CO₂ concentration → MH-Z19B, Senseair

- Light intensity → BH1750 (Lux), Apogee PAR

- Leaf wetness → Leaf Wetness Sensor

- Airflow → Air velocity sensors, fan RPM sensors

4.2 Soil & Root Zone Sensors

- Soil moisture → Capacitive soil moisture sensor

- Soil pH → pH sensor

- Nutrient concentration → EC sensor

5. AI → Insight → Action Mapping

5.1 Growth Rate Prediction

Inputs

- Temperature, light, CO₂, soil moisture, EC

AI Task

- Predict daily growth rate and biomass accumulation

Actions

- Adjust lighting duration

- Optimize nutrient delivery

5.2 Disease Prediction

Inputs

- Humidity duration

- Leaf wetness

- Temperature variation

- Camera images

AI Task

- Early disease risk classification

Actions

- Increase ventilation

- Reduce humidity

- Trigger preventive alerts

5.3 Sensor Anomaly Detection

Inputs

- Continuous sensor streams

AI Task

- Detect faulty sensors or abnormal spikes

Actions

- Ignore corrupted data

- Switch to fallback control logic

5.4 Harvest Time Prediction

Inputs

- Growth stage

- Vision-based ripeness indicators

AI Task

- Predict optimal harvest window

Actions

- Adjust final-stage irrigation and lighting

- Schedule labor and logistics

6. Controllable Greenhouse Outputs

Climate Control

- Ventilation fans (ON/OFF / variable speed)

- Exhaust vents

- Cooling pads

- Heaters

Light Control

- LED grow lights

- Photoperiod scheduling

Water & Nutrients

- Irrigation timing

- Fertigation EC control

Humidity Control

- Dehumidifiers

- Vent timing

CO₂ Management

- CO₂ injection

7. Human–Machine Interaction (Critical Design Principle)

Core Rule

AI never communicates directly with workers.

- AI → Machines

- Machines → People (via non-verbal signals)

Communication Channels

- 🔴 Light

- 🔊 Sound

- 🟢 Color

- 🔄 Machine movement

- 👋 Supervisor signals

Universal One-Look Rule

- 🟢 Green → OK, do nothing

- 🟡 Yellow → Caution

- 🔴 Red → Stop and inform supervisor

Symbol Rule

- 💧 Water

- 🌡️ Heat

- 🍅🍓 Harvest

- 🌿 Plant health

- 🧤 Safety

All AI decisions (ventilation, irrigation, humidity, lighting, disease alerts, harvest, fertilizer) are converted into simple physical signals, making the system usable by illiterate workers.

8. Data Pipeline & Software Stack

Data Ingestion

- MQTT broker: Eclipse Mosquitto

- Python client: paho-mqtt

- Node-RED (optional)

Storage

- InfluxDB or TimescaleDB

- Python clients: influxdb-client, psycopg2

Processing & Feature Engineering

- pandas, numpy

- scipy (filters)

- statsmodels (time series)

Computed features include:

- VPD

- Dew point

- Leaf wetness proxy

- Rolling humidity and airflow durations

- Estimated resource consumption (energy, water, CO₂)

9. Stage-Aware Target Profiles

Stages:

- Vegetative

- Flowering

- Fruiting

- Harvest

Stage detection methods:

- Rule-based (days after transplant)

- Vision-based

Stage configuration stored in YAML/JSON:

- Temperature, RH/VPD, CO₂ targets

- Soil moisture, EC, pH

- Photoperiod and light intensity

- Disease guardrails

- Resource priority weights

10. Disease Risk Index & Risk Budget

- Disease Risk Index scaled from 0–100

- Inputs:

  - RH > 85% duration

  - Leaf wetness duration

  - Temperature in disease-favorable band

  - Low airflow duration

- Risk budgets limit:

  - Maximum allowed wetness or humidity per hour/night

- If risk budget is nearly exhausted, the controller prioritizes drying actions

11. Forecasting Models (30–120 Minutes)

ML Models

- XGBoost

- RandomForest

Forecasts include:

- Temperature, RH/VPD, CO₂

- Soil moisture

- Leaf wetness probability

- Disease risk

Actuator states are included as disturbance variables.

12. Digital Twin & MPC-Style Control

The digital twin predicts greenhouse response to actions using a hybrid grey-box model.

State variables:

- Temperature, RH, CO₂

- Soil moisture, EC

- Leaf wetness

Control loop (every ~5 minutes):

- Read sensors

- Clean data and compute features

- Detect growth stage

- Compute disease risk and remaining budget

- Forecast next 30–120 minutes

- Generate candidate action sequences

- Simulate in digital twin

- Score using objective function

- Apply best first action

- Validate and log

13. Global Optimization Objective

The system minimizes:

- Tracking error (deviation from targets)

- Resource usage (energy, water, fertilizer, CO₂)

- Disease risk

- Actuator wear

Weights are stage-dependent, prioritizing quality and disease prevention during flowering and fruiting.

14. Safety Constraints

Non-negotiable guardrails:

- Temperature limits

- Night humidity limits

- CO₂ safety limits

- Hardware protection rules

- CO₂ injection disabled when vents are open

15. Reinforcement Learning (Optional Advanced Layer)

- RL trained inside the digital twin

- Libraries:

  - gymnasium

  - stable-baselines3 (PPO, SAC)

- RL proposes actions

- Safety shield enforces constraints before deployment

16. Modeling Strategy Summary

- CNN + RNN

  - Disease detection and progression

  - Growth stage prediction

- XGBoost / Random Forest

  - Environmental forecasting

  - Resource estimation

  - Anomaly detection

  - Regression tasks

17. External Datasets & Repositories Used

GitHub

LaboroTomato

https://github.com/laboroai/LaboroTomato

- Tomato growth stage classification

- Curated tomato growth stage image dataset

Kaggle Datasets

- Greenhouse Plant Growth Metrics
https://www.kaggle.com/datasets/adilshamim8/greenhouse-plant-growth-metrics

- Tabular time-series greenhouse and growth data

- Ripe Strawberries Detection
https://www.kaggle.com/datasets/trainingdatapro/ripe-strawberries-detection

- Labeled strawberry images for ripeness detection

- Tomato Plant Stages Dataset
https://www.kaggle.com/datasets/arjunsudheer326/tomato-plant-stages-dataset

- Labeled tomato growth stage images

- Smart Agriculture Dataset
https://www.kaggle.com/datasets/chaitanyagopidesi/smart-agriculture-dataset/data

- Tabular sensor-based smart agriculture data (temperature, humidity, soil moisture, etc.)

- PlantVillage Dataset
https://www.kaggle.com/datasets/arjuntejaswi/plant-village

- Large-scale labeled plant leaf images covering healthy and diseased conditions across multiple crops

18. Final Outcome

AgriTwin-GH delivers:

- Maximum yield

- Minimum resource waste

- Early disease prevention

- Safe, intuitive operation for workers

- A scalable, AI-driven digital twin greenhouse system